| TDI-Brooks International |  |  | Enter in NS5 Quality module as a drill & in ship’s log |  |  | Vessel: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NS5 Drill # |  |  |  | Emergency Drill |  |  |  |  |  |
|  |  |  |  | # Foundering/ Grounding |  |  |  |  |  |
| Date: |  |  |  |  |  |  | Start Time: |  |  |
| Conducted By (print name): |  |  |  |  |  |  | End Time: |  |  |

| No. | NAME | Initials | No. | NAME | Initials |
| --- | --- | --- | --- | --- | --- |
| 1. |  |  | 16. |  |  |
| 2. |  |  | 17. |  |  |
| 3. |  |  | 18. |  |  |
| 4. |  |  | 19. |  |  |
| 5 |  |  | 20. |  |  |
| 6 |  |  | 21. |  |  |
| 7 |  |  | 22. |  |  |
| 8. |  |  | 23. |  |  |
| 9. |  |  | 24. |  |  |
| 10. |  |  | 25. |  |  |
| 11. |  |  | 26. |  |  |
| 12. |  |  | 27. |  |  |
| 13. |  |  | 28 |  |  |
| 14. |  |  | 29. |  |  |
| 15. |  |  | 30. |  |  |
| Suggested Topics |  |  |  |  |  |
| All hands sign in above – Topics covered include: Grounding may occur on submerged structure including sand bars. Should the vessel become grounded the following actions are to be taken: 1. On the bridge : - stop engine, inform engineer on watch that the vessel is grounded - Call master - Inform any nearby vessels or structures of grounding - Use appropriate signals (day-3 black balls & night- 3 red mast lights) - Reference the chart for the type of sea bottom (mud, sand, rock) - Check tides - Determine danger of sinking, Master to signal abandon ship when it appears necessary 2. On the deck : - visually inspect the vessel, check all compartments for damage/leakage and take sounding around the vessel to find which side is grounded (port, stbd, bow, stern) 3. Engine room : - take sounding of ballast and bunker tank compartments adjacent to hull Compare present sounding to departure sounding Determine risk of flooding Determine risk of oil spill Master and Chief Engineer to evaluate what, if any damage resulted and the best procedure for refloating the vessel or contacting emergency response resources for assistance. |  |  |  |  |  |
| Summarize today’s drill scenario: Summarize crew performance: Lessons learned: |  |  |  |  |  |