| TDI-Brooks International |  |  | Enter in NS5 Quality module as a drill & in ship’s log |  |  | Vessel: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NS5 Drill # |  |  |  | Emergency Drill |  |  |  |  |  |
|  |  |  |  | # Loss of Steering |  |  |  |  |  |
| Date: |  |  |  |  |  |  | Start Time: |  |  |
| Conducted By (print name): |  |  |  |  |  |  | End Time: |  |  |

| No. | NAME | Initials | No. | NAME | Initials |
| --- | --- | --- | --- | --- | --- |
| 1. |  |  | 14. |  |  |
| 2. |  |  | 15. |  |  |
| 3. |  |  | 16. |  |  |
| 4. |  |  | 17. |  |  |
| 5 |  |  | 18. |  |  |
| 6 |  |  | 19. |  |  |
| 7 |  |  | 20. |  |  |
| 8. |  |  | 21. |  |  |
| 9. |  |  | 22. |  |  |
| 10. |  |  | 23. |  |  |
| 11. |  |  | 24. |  |  |
| 12. |  |  | 25. |  |  |
| 13. |  |  | 26. |  |  |
| Suggested Topics |  |  |  |  |  |
| Vessel engineering and bridge crew should verify protocols in the event the vessel loses steering |  |  |  |  |  |
| Protocols to be followed differ for each vessel. Refer to SOP-007P for vessel specifics. |  |  |  |  |  |
| General protocols involve the following: |  |  |  |  |  |
| - Notify the engineering department and master immediately. |  |  |  |  |  |
| - Use the engines to maneuver the vessel away from danger. |  |  |  |  |  |
| - Inform any nearby vessels or structures by VHF (AIS) |  |  |  |  |  |
| - Use the appropriate signals (Day – two black balls on yard arm, night- turn off white mast light and turn on 2 reds mast lights ) to let vessel know steering ability is compromised (NUC) |  |  |  |  |  |
| - Contact the engine room, and have the engineering staff visually inspect the steering pump motors. |  |  |  |  |  |
| - If the motor are not operational, then the engineers will inspect both the breaker of the power supply panel and the motor controller (contactor) switch boxes. |  |  |  |  |  |
| - The engineer will attempt to engage the motor manually using the manual bypass on the motor controller switch boxes. |  |  |  |  |  |
| - If the engineer is able to engage either of the two steering pump motors, test if the steering has been restored at the helm, if not continue with the following steps. |  |  |  |  |  |
| - Should one of the steering pump motor be operational and steering control is still absent on the helm then via communication lines the master or mate will issue commands to engineers to manually operate the steering solenoids (Emergency Steering). |  |  |  |  |  |
| 12. Should the pump motors be inoperable, instruct the engineering staff to manually center the rudder and use the vessel’s engine to control vessel movement. |  |  |  |  |  |

Summarize scenario, crew performance and lessons learned: