I made the following control interlock wiring changes in the winch control functions.

The remote control for the traction winch will not drive the traction winch nor 	release the traction winch brake unless the remote control console has been 	enabled.

The remote console cannot be enabled if the remote control handle is out of 	neutral.

The remote control console cannot be enabled unless the storage winch is 	in the 	“automatic” mode.

If the local traction control handle is moved out of neutral then the remote control 	console is disabled but the storage winch will stay in the “automatic” mode. The 	remote must be enabled from the local console in order to be functional again.

If the storage winch control is moved out of neutral, the storage winch is taken out 	of “automatic” mode and the remote control console is disabled. In order to use 	the remote control console the storage winch must  be put back into” automatic” 	mode and the remote re-enabled.

The above changes were made for safety reasons. The winch should now conform to Dynacons typical operator control interlock safety standards.

Best, 	Mike Cooke

1905 Pinewood

Bryan, TX 77807	Cell phone # 979 218-1825