4043 Hollister

DT MARINE  PRODUCTS,INC.

Houston, TX  77080

Phone  (713)460-1400

Fax      (713)460-3585

dtmarine@sbcglobal.net

SPECIFICATIONS

MODEL DT3040-EHLW ELECTRO-HYDRAULIC WINCH

DIMENSIONS:

WIDTH                             78 INCHES

LENGTH                          58 INCHES(INCLUDES LEVEL-WIND)

HEIGHT                           58 INCHES(INCLUDES LEVEL-WIND)

DRUM DIAMETER         24 INCHES

FLANGE DIAMETER     46 INCHES

DRUM WIDTH                36 INCHES

CAPACITY:

5,000 FEET OF .75” DIA. CABLE ( LEAVES 2” CLEAR FLANGE)

CONSTRUCTION:

ALL WELDED STEEL WITH STAINLESS STEEL HARDWARE

COATING:

SYSTEM IS BLASTED TO NEAR WHITE METAL AND PAINTED WITH

THREE-COAT EPOXY SYSTEM, ZINC PRIMER, TIECOAT, TOP COAT

EPOXY

BEARINGS;

SEALED, SELF-ALIGNING BALL BEARINGS

ELECTRIC MOTOR:

40HP, 460, 60HZ THREE PHASE, TOTALLY  ENCLOSED

FAN COOLED

PUMP:

CLOSED LOOP AXIAL PISTON SUNDSTRAND.

HYDRAULIC MOTOR:

AXIAL PISTON SUNDSTRAND

LINE PULL & SPEED:

10,000 LBS BARE DRUM RATING @ 0-80 FPM

SYSTEM:

CLOSED LOOP, MOTOR COUPLED TO DRUM VIA PLANETARY

GEAR  REDUCER AND FAILSAFE BRAKE PROVIDED. MULTIPLE

DISC  FAILSAFE BRAKE BETWEEN MOTOR AND GEARBOX.

RESERVOIR:

INCORPORATED INTO BASE OF THE WINCH

CONTROLS:

SELF-CENTERING, SINGLE LEVER JOY STICK PROVIDES LINEAR

VARIABLE SPEED OPERATION.  PRESSURE GUAGE,  POWER

ON/OFF SWITCHES AND NEMA 4X MOTOR STARTER ENCLOSURE

ARE ALL SAFELY MOUNTED FOR  EASY OPERATION.

BRAKE SYSTEM:

FAIL-SAFE BRAKE IS ACTUATED FROM CENTER POSITION OF

“JOY STICK”.  LOSS OF HYDRAULIC PRESSURE OR LOSS OF

ELECTRIC POWER, SYSTEM INCLUDES MANUALLY  OPERATED 3”

STAINLESS STEEL BAND BRAKE AS A  STANDARD SAFETY

FEATURE

LEVEL-WIND SYSEM:

HYDROACTIVE CABLE GUIDE SYSTEM, FRONT MOUNTED BALL

SCREW TYPE, AUTOMATIC BI-DIRECTIONAL ANGLE SENSING

SYSTEM WITH MANUALLY OPERATED OVERRIDE

HANDLING:

LOAD TESTED 4 POINT LIFT EYES/FORK LIFT TUBES.

WEIGHT:

THE APPROXIMATE WEIGHT FOR WINCH WITHOUT

OIL OR CABLE IS 5,500 LBS.

DT MARINE PROVIDES:

WINCH ENGINEERING SERVICES, ALUMINUM OR STEEL CONSTRUCTION, EXPLOSION PROOF MODELS, LEVEL-WINDS, CABLE SHEAVES, CABLES, SLIP-

RINGS, REMOTE CONTROL OPERTIONS AND COMPLETE CUSTOMER SERVICE AFTER THE SALE.  FOR PRODUCTS AND SERVICES CONTACT:  DARRELL TROVILLE @ (713)460-1400