TORVANE INSTRUCTIONS

Do TORVANE first – helps you accurately set up MINIVANE later.

Torvane has four parts:

Red top TORVANE with allen wrenche(s) and two alternate size heads.

**You want to disturb the mud as little as possible for accurate readings.**

- Choose the correct Torvane head size.  The wetter the mud the wider the blade/ the thicker the mud the thinner the vane.

- Attach the correct size blade and tighten by inserting the allen screw into the small hole in the side of the blade.

- Make sure the Torvane head is set to zero- or adjusted all the way back to the stopping pin.  Record the number as your starting reading.

- Insert the blades of the torvane into the mud- just enough to cover the ¼ inch blades.  Hold as still as possible.

- Slowly turn the head in a clockwise direction, keeping the blades as level as possible.  At some point, the mud will “give” or “break” and the torvane will start to spin freely.

- Stop turning as soon as you feel the mud “give” or “break”.  Read the top of the Torvane and record the resistance reading.

- Record the end reading.  If the start reading was not zero, then calculate the difference between the start and end readings.

  - The end reading needs to be between________and_______.

- If you had too much resistance, change to a smaller blade size.  If too much resistance, change to a larger head size and try again in an undisturbed area of mud.

**Using the Torvane to estimate the Minivane spring resistance.**

- Take the end reading from the Torvane and insert it into the spreadsheet in the column titled TV.

- Enter the size blade used into the column titled Blade Size.  Choices are small (1/2 by 12 blade), medium (1” by ½ “ blade) and large (1 x 1 inch blade).

- The computer will calculate the TVR or the Torvane resistance in the column of the same name.

- Take the resistance number and insert it into the Minivane spreadsheet, which will indicate which minivane spring type (resistance level) to use for the undisturbed Minivane.