MSDS Number: S4106 * * * * * Effective Date: 02/04/09 * * * * * Supercedes: 01/31/07

# SODIUM HYPOCHLORITE SOLUTION (Liquid Bleach)

## 1. Product Identification

Synonyms: Bleach; hypochlorous acid, sodium salt; soda bleach; sodium oxychloride 
CAS No.: 7681-52-9 
Molecular Weight: 74.44 
Chemical Formula: NaOCl 
Product Codes: 9416, P005

## 2. Composition/Information on Ingredients

Ingredient                                CAS No         Percent        Hazardous

---------------------------------------   ------------   ------------   ---------

Sodium Hypochlorite (as NaOCl)            7681-52-9           5%           Yes

Water                                     7732-18-5           95%          No

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
WARNING! HARMFUL IF SWALLOWED OR INHALED. CAUSES IRRITATION TO EYES AND RESPIRATORY TRACT. CAUSES SUBSTANTIAL BUT TEMPORARY EYE INJURY. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 2 - Moderate 
Flammability Rating: 0 - None 
Reactivity Rating: 1 - Slight 
Contact Rating: 3 - Severe 
Lab Protective Equip: GOGGLES & SHIELD; LAB COAT & APRON; VENT HOOD; PROPER GLOVES 
Storage Color Code: Green (General Storage) 
----------------------------------------------------------------------------------------------------------- 
Potential Health Effects 
---------------------------------- 
Inhalation: 
May cause irritation to the respiratory tract, (nose and throat); symptoms may include coughing and sore throat. 
Ingestion: 
May cause nausea, vomiting. 
Skin Contact: 
May irritate skin. 
Eye Contact: 
Contact may cause severe irritation and damage, especially at higher concentration. 
Chronic Exposure: 
A constant irritant to the eyes and throat. Low potential for sensitization after exaggerated exposure to damaged skin. 
Aggravation of Pre-existing Conditions: 
Persons with impaired respiratory function, or heart disorders (or disease) may be more susceptible to the effects of the substance.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention immediately. 
Ingestion: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. Get medical attention immediately. 
Skin Contact: 
Immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Get medical attention immediately. Wash clothing before reuse. Thoroughly clean shoes before reuse. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately. 

Note to Physician: 
Consider oral administration of sodium thiosulfate solutions if sodium hypochlorite is ingested. Do not administer neutralizing substances since the resultant exothermic reaction could further damage tissue. Endotracheal intubation could be needed if glottic edema compromises the airway. For individuals with significant inhalation exposure, monitor arterial blood gases and chest x-ray.

## 5. Fire Fighting Measures

Fire: 
Not considered to be a fire hazard. Substance releases oxygen when heated, which may increase the severity of an existing fire. Containers may rupture from pressure build-up. 
Explosion: 
This solution is not considered to be an explosion hazard. Anhydrous sodium hypochlorite is very explosive. 
Fire Extinguishing Media: 
Use any means suitable for extinguishing surrounding fire. Use water spray to cool fire-exposed containers, to dilute liquid, and control vapor. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Isolate hazard area. Keep unnecessary and unprotected personnel from entering. Contain and recover liquid when possible. Collect liquid in an appropriate container or absorb with an inert material (e. g., vermiculite, dry sand, earth), and place in a chemical waste container. Do not use combustible materials, such as saw dust. Do not flush to sewer! US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802.

## 7. Handling and Storage

Keep in a tightly closed container, stored in a cool, dry, ventilated area. Protect against physical damage. Isolate from incompatible substances. Containers of this material may be hazardous when empty since they retain product residues (vapors, liquid); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
Sodium Hypochlorite:
AIHA (WEEL) - STEL - 2 mg/m3

-OSHA Permissible Exposure Limit (PEL):
0.5 ppm (TWA), 1 ppm (STEL) as Chlorine

-ACGIH Threshold Limit Value (TLV):
1 ppm (TWA), 3 ppm (STEL) as Chlorine 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
If the exposure limit is exceeded and engineering controls are not feasible, a full facepiece respirator with an acid gas cartridge may be worn up to 50 times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is lowest. For emergencies or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator. WARNING: Air purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Use chemical safety goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area.

## 9. Physical and Chemical Properties

Appearance: 
Colorless to yellowish liquid. 
Odor: 
Chlorine-like odor. 
Solubility: 
100% in water. 
Density: 
1.07 - 1.14 
pH: 
9 - 10 (neutral solution-no excess sodium hydroxide) 
% Volatiles by volume @ 21C (70F): 
ca. 95 
Boiling Point: 
40C (104F) Decomposes slightly 
Melting Point: 
-6C (21F) 
Vapor Density (Air=1): 
No information found. 
Vapor Pressure (mm Hg): 
17.5 @ 20C (68F) 
Evaporation Rate (BuAc=1): 
No information found.

## 10. Stability and Reactivity

Stability: 
Slowly decomposes on contact with air. Rate increases with the concentration and temperature. Exposure to sunlight accelerates decomposition. Sodium hypochlorite becomes less toxic with age. 
Hazardous Decomposition Products: 
Emits toxic fumes of chlorine when heated to decomposition. Sodium oxide at high temperatures. 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Ammonia (chloramine gas may evolve),amines, ammonium salts, aziridine, methanol, phenyl acetonitrile, cellulose, ethyleneimine, oxidizable metals, acids, soaps, and bisulfates. 
Conditions to Avoid: 
Light, heat, incompatibles.

## 11. Toxicological Information

No LD50/LC50 information found relating to normal routes of occupational exposure. Investigated as a tumorigen and mutagen. Irritation data: eye, rabbit, 10 mg - Moderate 

Results from Corrositex® Testing: > 60 minutes, non-corrosive.

--------\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                             Known    Anticipated    IARC Category

------------------------------------   -----    -----------    -------------

Sodium Hypochlorite (as NaOCl)          No          No              3

(7681-52-9)

Water (7732-18-5)                       No          No            None

## 12. Ecological Information

Environmental Fate: 
No information found. 
Environmental Toxicity: 
No information found.

## 13. Disposal Considerations

Dilute with water and flush to sewer if local ordinances allow, otherwise, whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Not regulated.

## 15. Regulatory Information

--------\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                       TSCA  EC   Japan  Australia

-----------------------------------------------  ----  ---  -----  ---------

Sodium Hypochlorite (as NaOCl) (7681-52-9)        Yes  Yes   Yes      Yes

Water (7732-18-5)                                 Yes  Yes   Yes      Yes

--------\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                       Korea  DSL   NDSL  Phil.

-----------------------------------------------  -----  ---   ----  -----

Sodium Hypochlorite (as NaOCl) (7681-52-9)        Yes   Yes   No     Yes

Water (7732-18-5)                                 Yes   Yes   No     Yes

--------\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                                 RQ    TPQ     List  Chemical Catg.

-----------------------------------------  ---   -----   ----  --------------

Sodium Hypochlorite (as NaOCl) (7681-52-9) No    No      No         No

Water (7732-18-5)                          No    No      No         No

--------\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                 CERCLA     261.33     8(d)

-----------------------------------------  ------     ------    ------

Sodium Hypochlorite (as NaOCl)             100        No         No

(7681-52-9)

Water (7732-18-5)                          No         No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: No   Fire: No  Pressure: No

Reactivity: No          (Mixture / Liquid)

Australian Hazchem Code: None allocated. 
Poison Schedule: S5 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 2 Flammability: 0 Reactivity: 1 
Label Hazard Warning: 
WARNING! HARMFUL IF SWALLOWED OR INHALED. CAUSES IRRITATION TO EYES AND RESPIRATORY TRACT. CAUSES SUBSTANTIAL BUT TEMPORARY EYE INJURY. 
Label Precautions: 
Avoid contact with eyes, skin and clothing.
Avoid breathing mist.
Keep container closed.
Use with adequate ventilation.
Wash thoroughly after handling. 
Label First Aid: 
If swallowed, DO NOT INDUCE VOMITING. Give large quantities of water. Never give anything by mouth to an unconscious person. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse. In all cases get medical attention immediately. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 11. 
Disclaimer: 
************************************************************************************Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************************Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)