|  | Metal Brite (Catalog # IC-107) Technical Data - MSDS |
| --- | --- |

Description:
METAL BRITE is an acidic liquid cleaner for aluminum, stainless, painted surfaces and concrete. This product will brighten aluminum and stainless as it cleans.

Application:

| Refining & Petrochemical | Concrete and painted surfaces, aluminum insulation |
| --- | --- |
| Transportation | Stainless steel and aluminum trailers, aluminum and stainless tankers. |
| Pulp and Paper Industry | Floors hoods, stainless portions of fourdrinier. |
| Rail Car Industry | Tank and hopper cars contaminated with heavy road film or oxidized paint. |
| Soils Removed | Light oils, rust and other stains, algae, water scale, oxide films. |

Directions for use:
Dilute one part METAL BRITE with up to 10 parts water for brightening aluminum or stainless for removal of rust and other stains of algae, dilute 1:2 to 1:5, depending on severity of soil. For rail cars, dilute 1:1 or use concentrated depending on the severity of the road film or chalking.

Physical Properties:

| Appearance | Clear colorless liquid with medium viscosity |
| --- | --- |
| Specific Gravity | 1.3 |
| Flash Point | None |
| Solubility in Water | 100% |
| Foaming Tendency | Moderate |

Precautions:
Do not use on anodized aluminum or galvanized parts. Do not use on vehicle windshields or windows. Use protective clothing. Personnel should wear rubber gloves, rubber boots, face shield or monogoggles. DO NOT INGEST.

Standard Packaging: 55 Gallon Closed-head drum. Bulk.

FOR INDUSTRIAL USE ONLY.