|  | Common Name:; EXTREME DUTY SILICONE (AEROSOL) 03030. Manufacturer:; CRC. MSDS Revision Date:; 1/25/2008. Grainger Item Number(s):; 3EED2, 4ZF08. Manufacturer Model Number(s):. MSDS Table of Contents Click the desired link below to jump directly to that section in the MSDS. SECTION 1: PRODUCT & COMPANY IDENTIFICATION. SECTION 2: HAZARDS IDENTIFICATION. SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS. SECTION 4: FIRST AID MEASURES. SECTION 5: FIRE-FIGHTING MEASURES. SECTION 6: ACCIDENTAL RELEASE MEASURES. SECTION 7: HANDLING AND STORAGE. SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION. SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES. SECTION 10: STABILITY AND REACTIVITY. SECTION 11: TOXICOLOGICAL INFORMATION. SECTION 12: ECOLOGICAL INFORMATION. SECTION 13: DISPOSAL CONSIDERATIONS. SECTION 14: TRANSPORT INFORMATION. SECTION 15: REGULATORY INFORMATION. SECTION 16: OTHER INFORMATION. CRC(R*) MATERIAL SAFETY DATA SHEET SECTION 1: PRODUCT & COMPANY IDENTIFICATION. PRODUCT NAME: EXTREME DUTY SILICONE (AEROSOL) PRODUCT NUMBER (S): 03030 MANUFACTURED BY: CRC INDUSTRIES, INC. 885 LOUIS DRIVE WARMINSTER, PA 18974 WWW.CRCINDUSTRIES.COM GENERAL INFORMATION: (215) 674-4300 TECHNICAL ASSISTANCE: (800) 521-3168 CUSTOMER SERVICE: (800) 272-4620 24-HR EMERGENCY (CHEMTREC): (800) 424-9300 SECTION 2: HAZARDS IDENTIFICATION. EMERGENCY OVERVIEW: APPEARANCE & ODOR: CLEAR, WATER-WHITE LIQUID WITH MILD SOLVENT ODOR DANGER: EXTREMELY FLAMMABLE. HARMFUL OR FATAL IF SWALLOWED. CONTENTS UNDER PRESSURE. AS DEFINED BY OSHA'S HAZARD COMMUNICATION STANDARD, THIS PRODUCT IS HAZARDOUS. POTENTIAL HEALTH EFFECTS: EYE: MAY CAUSE MILD IRRITATION INCLUDING STINGING AND REDNESS, BUT DOES NOT INJURE EYE. SKIN: SINGLE, BRIEF EXPOSURES MAY CAUSE MILD IRRITATION. FREQUENT OR PROLONGED CONTACT MAY CAUSE MORE SEVERE IRRITATION, DEFATTING OF THE SKIN, AND DERMATITIS. INHALATION: HIGH VAPOR CONCENTRATIONS ARE IRRITATING TO THE RESPIRATORY TRACT AND MAY CAUSE HEADACHES, DIZZINESS, ANESTHESIA, DROWSINESS, UNCONSCIOUSNESS AND OTHER CENTRAL NERVOUS SYSTEM EFFECTS, INCLUDING DEATH. MAY CAUSE PERIPHERAL NERVOUS SYSTEM DISORDER AND/OR DAMAGE. INGESTION: LOW ORDER OF TOXICITY BY INGESTION. MAIN HAZARD IS ASPIRATION INTO THE LUNGS DURING SWALLOWING OR VOMITING. SMALL AMOUNTS ASPIRATED INTO THE RESPIRATORY SYSTEM MAY CAUSE BRONCHOPNEUMONIA OR PULMONARY ADEMA, POSSIBLE PROGRESSING TO DEATH. CHRONIC EFFECTS: OVEREXPOSURE TO n-HEXANE MAY CAUSE PROGRESSIVE AND POTENTIALLY IRREVERSIBLE DAMAGE TO THE PERIPHERAL NERVOUS SYSTEM, PARTICULARLY IN THE ARMS AND LEGS. TARGET ORGANS: CENTRAL NERVOUS SYSTEM, PERIPHERAL NERVOUS SYSTEM, RESPIRATORY SYSTEM MEDICAL CONDITIONS AGGRAVATED BY EXPOSURE: SKIN AND RESPIRATORY CONDITIONS SEE SECTION 11 FOR TOXICOLOGY AND CARCINOGENICITY INFORMATION ON PRODUCT INGREDIENTS. SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS. COMPONENT CAS NUMBER % BY WT. HEXANE ISOMERS VARIOUS 55 - 65 n-HEXANE 110-54-3 <2 SILICONE FLUID PROPRIETARY 2 - 5 1,1-DIFLUOROETHANE (HFC-152A) 75-37-6 30 - 40 SECTION 4: FIRST AID MEASURES. EYE CONTACT: IMMEDIATELY FLUSH WITH PLENTY OF WATER FOR 15 MINUTES. CALL A PHYSICIAN IF IRRITATION PERSISTS. SKIN CONTACT: REMOVE CONTAMINATED CLOTHING AND WASH AFFECTED AREA WITH SOAP AND WATER. CALL A PHYSICIAN IF IRRITATION PERSISTS. WASH CONTAMINATED CLOTHING PRIOR TO RE-USE. INHALATION: REMOVE PERSON TO FRESH AIR. KEEP PERSON CALM. IF NOT BREATHING, GIVE ARTIFICIAL RESPIRATION. IF BREATHING IS DIFFICULT GIVE OXYGEN. CALL A PHYSICIAN. INGESTION: DO NOT INDUCE VOMITING. CONTACT A PHYSICIAN IMMEDIATELY. NOTE TO PHYSICIANS: TREAT SYMPTOMATICALLY. GASTRIC LAVAGE USING A CUFFED ENDOTRACHEAL TUBE MAY BE PERFORMED AT YOUR DISCRETION. SECTION 5: FIRE-FIGHTING MEASURES. FLAMMABLE PROPERTIES: THIS PRODUCT IS EXTREMELY FLAMMABLE IN ACCORDANCE WITH AEROSOL FLAMMABILITY DEFINITIONS (16 CFR 1500.3(C)(6)). FLASH POINT: <0 F (TCC) AUTOIGNITION TEMPERATURE: 489 F UPPER EXPLOSIVE LIMIT: 9.0 LOWER EXPLOSIVE LIMIT: 1.7 SUITABLE EXTINGUISHING MEDIA: CLASS B FIRE EXTINGUISHERS, DRY CHEMICAL, FOAM OR CO2 PRODUCTS OF COMBUSTION: FUMES, SMOKE AND CARBON MONOXIDE PROTECTION OF FIRE-FIGHTERS: FIREFIGHTERS SHOULD WEAR SELF-CONTAINED, NIOSH-APPROVED BREATHING APPARATUS FOR PROTECTION AGAINST SUFFOCATION AND POSSIBLE TOXIC DECOMPOSITION PRODUCTS. PROPER EYE AND SKIN PROTECTION SHOULD BE PROVIDED. USE WATER FOG OR SPRAY TO KEEP FIRE-EXPOSED CONTAINERS COOL AND TO KNOCK DOWN VAPORS WHICH MAY RESULT FROM PRODUCT DECOMPOSITION. DO NOT SPRAY WATER DIRECTLY ON FIRE; PRODUCT WILL FLOAT AND COULD BE REIGNITED ON SURFACE OF WATER. SECTION 6: ACCIDENTAL RELEASE MEASURES. PERSONAL PRECAUTIONS: USE PERSONAL PROTECTION RECOMMENDED IN SECTION 8. ENVIRONMENTAL PRECAUTIONS: TAKE PRECAUTIONS TO PREVENT CONTAMINATION OF GROUND AND SURFACE WATERS. DO NOT FLUSH INTO SEWERS OR STORM DRAINS. METHODS FOR CONTAINMENT & CLEAN-UP: DIKE AREA TO CONTAIN SPILL. REMOVE ALL SOURCES OF IGNITION. VENTILATE THE AREA WITH FRESH AIR. IF IN CONFINED SPACE OR LIMITED AIR CIRCULATION AREA, CLEAN-UP WORKERS SHOULD WEAR APPROPRIATE RESPIRATORY PROTECTION. RECOVER OR ABSORB SPILLED MATERIAL USING AN ABSORBENT DESIGNED FOR CHEMICAL SPILLS. PLACE USED ABSORBENTS INTO PROPER WASTE CONTAINERS. SECTION 7: HANDLING AND STORAGE. HANDLING PROCEDURES: USE PROPER GROUNDING AND BONDING PROCEDURES FOR TRANSFERRING MATERIALS. DO NOT USE PRODUCT NEAR ANY SOURCE OF IGNITION. DO NOT TOUCH CONTAINER TO ELECTRICAL SOURCES AS CONTAINER WILL CONDUCT ELECTRICITY. AVOID CONTACT WITH EYES AND SKIN. AVOID BREATHING VAPORS. STORAGE PROCEDURES: STORE IN A COOL DRY AREA OUT OF DIRECT SUNLIGHT. AEROSOL CANS MUST BE MAINTAINED BELOW 120 F TO PREVENT CANS FROM RUPTURING. AEROSOL STORAGE LEVEL: III SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION. EXPOSURE GUIDELINES: COMPONENT OSHA ACGIH OTHER UNIT TWA STEL TWA STEL TWA SOURCE HEXANE ISOMERS 500(V) 1000(V) 500 1000 NE PPM n-HEXANE 500 NE 50(S) NE NE PPM SILICONE FLUID NE NE NE NE NE 1,1-DIFLUOROETHANE NE NE NE NE 1000 AIHA PPM N.E. - NOT ESTABLISHED (C) - CEILING (S) - SKIN (V) - VACATED ENGINEERING CONTROLS: AREA SHOULD HAVE VENTILATION TO PROVIDE FRESH AIR. USE LOCAL EXHAUST TO PREVENT ACCUMULATION OF VAPORS. USE MECHANICAL MEANS IF NECESSARY TO MAINTAIN VAPOR LEVELS BELOW THE EXPOSURE GUIDELINES. IF WORKING IN A CONFINED SPACE, FOLLOW APPLICABLE OSHA REGULATIONS RESPIRATORY PROTECTION: NONE REQUIRED FOR NORMAL WORK WHERE ADEQUATE VENTILATION IS PROVIDED. USE A NIOSH-APPROVED CARTRIDGE RESPIRATOR WITH AN ORGANIC VAPOR CARTRIDGE IF VAPORS EXCEED EXPOSURE LIMITS. USE A SELF-CONTAINED BREATHING APPARATUS IN CONFINED SPACES AND FOR EMERGENCIES. EYE/FACE PROTECTION: FOR NORMAL CONDITIONS, WEAR SAFETY GLASSES. WHERE THERE IS REASONABLE PROBABILITY OF LIQUID CONTACT, WEAR SPLASH-PROOF GOGGLES. SKIN PROTECTION: USE PROTECTIVE GLOVES SUCH AS NITRILE, PVC OR VITON. ALSO, USE FULL PROTECTIVE CLOTHING IF THERE IS PROLONGED OR REPEATED CONTACT OF LIQUID WITH SKIN. SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES. PHYSICAL STATE: LIQUID COLOR: CLEAR, WATER-WHITE ODOR: MILD SOLVENT SPECIFIC GRAVITY: 0.6699 INITIAL BOILING POINT: 140 F FREEZING POINT: <-76 F VAPOR PRESSURE: 160 MMHg @ 68 F VAPOR DENSITY: >1 (AIR = 1) EVAPORATION RATE: >1 (BUTYL ACETATE = 1) SOLUBILITY: NEGLIGIBLE IN WATER pH: NA VOLATILE ORGANIC COMPOUNDS: WT %: 60 G/L: 400.2 LBS./GAL: 3.34 SECTION 10: STABILITY AND REACTIVITY. STABILITY: STABLE CONDITIONS TO AVOID: SOURCES OF IGNITION, TEMPERATURE EXTREMES INCOMPATIBLE MATERIALS: STRONG OXIDIZERS HAZARDOUS DECOMPOSITION PRODUCTS: OXIDES OF CARBON POSSIBILITY OF HAZARDOUS REACTIONS: NO SECTION 11: TOXICOLOGICAL INFORMATION. LONG-TERM TOXICOLOGICAL STUDIES HAVE NOT BEEN CONDUCTED FOR THIS PRODUCT. THE FOLLOWING INFORMATION IS AVAILABLE FOR COMPONENTS OF THIS PRODUCT. ACUTE EFFECTS: COMPONENT TEST RESULT ROUTE SPECIES n-HEXANE LD50 28710 MG/KG ORAL RAT n-HEXANE LD50 3000 MG/KG DERMAL RABBIT n-HEXANE LC50 48000 PPM/4H INHALATION RAT CHRONIC EFFECTS: CARCINOGENICITY: COMPONENT RESULT OSHA NONE LISTED IARC NONE LISTED NTP NONE LISTED MUTAGENICITY: NO INFORMATION AVAILABLE SECTION 12: ECOLOGICAL INFORMATION. ECOLOGICAL STUDIES HAVE NOT BEEN CONDUCTED FOR THIS PRODUCT. THE FOLLOWING INFORMATION IS AVAILABLE FOR COMPONENTS OF THIS PRODUCT. ECOTOXICITY: n-HEXANE - 48 HR EC50 WATER FLEA: 3.87 MG/L 96 HR LC50 LEPOMIS MACROCHIRUS: 4.12 MG/L PERSISTENCE / DEGRADABILITY: NO INFORMATION AVAILABLE BIOACCUMULATION / ACCUMULATION: NO INFORMATION AVAILABLE MOBILITY IN ENVIRONMENT: NO INFORMATION AVAILABLE SECTION 13: DISPOSAL CONSIDERATIONS. DISPOSAL: THE DISPENSED LIQUID PRODUCT IS A RCRA HAZARDOUS WASTE FOR THE CHARACTERISTIC OF IGNITABILITY WITH A WASTE CODE OF D001 (SEE 40 CFR PART 261.20 - 261.33). AEROSOL CONTAINERS SHOULD BE EMPTIED AND DEPRESSURIZED BEFORE DISPOSAL. EMPTY CONTAINERS MAY BE RECYCLED. ANY LIQUID PRODUCT SHOULD BE MANAGED AS A HAZARDOUS WASTE. ALL DISPOSAL ACTIVITIES MUST COMPLY WITH FEDERAL, STATE AND LOCAL REGULATIONS. LOCAL REGULATIONS MAY BE MORE STRINGENT THAN STATE OR NATIONAL REQUIREMENTS. SECTION 14: TRANSPORT INFORMATION. PROPER SHIPPING DESCRIPTION: US DOT (GROUND): CONSUMER COMMODITY, ORM-D SPECIAL PROVISIONS: NONE SECTION 15: REGULATORY INFORMATION. U.S. FEDERAL: TOXIC SUBSTANCES CONTROL ACT (TSCA): ALL INGREDIENTS ARE EITHER LISTED ON THE TSCA INVENTORY OR ARE EXEMPT. COMPREHENSIVE ENVIRONMENTAL RESPONSE, COMPENSATION AND LIABILITY ACT (CERCLA): REPORTABLE QUANTITIES (RQ'S) EXIST FOR THE FOLLOWING INGREDIENTS: n-HEXANE (5000 LBS) SPILLS OR RELEASES RESULTING IN THE LOSS OF ANY INGREDIENT AT OR ABOVE ITS RQ REQUIRE IMMEDIATE NOTIFICATION TO THE NATIONAL RESPONSE CENTER (800-424-8802) AND TO YOUR LOCAL EMERGENCY PLANNING COMMITTEE. SUPERFUND AMENDMENTS REAUTHORIZATION ACT (SARA) TITLE III: SECTION 302 EXTREMELY HAZARDOUS SUBSTANCES (EHS): NONE SECTION 311/312 HAZARD CATEGORIES: FIRE HAZARD: YES REACTIVE HAZARD: NO RELEASE OF PRESSURE: YES ACUTE HEALTH HAZARD: YES CHRONIC HEALTH HAZARD: YES SECTION 313 TOXIC CHEMICALS: THIS PRODUCT CONTAINS THE FOLLOWING SUBSTANCES SUBJECT TO THE REPORTING REQUIREMENTS OF SECTION 313 OF TITLE III OF THE SUPERFUND AMENDMENTS AND REAUTHORIZATION ACT OF 1986 AND 40 CFR PART 372: n-HEXANE (1.8%) CLEAN AIR ACT: SECTION 112 HAZARDOUS AIR POLLUTANTS (HAPS): n-HEXANE STATE REGULATIONS: CALIFORNIA SAFE DRINKING WATER AND TOXIC ENFORCEMENT ACT (PROP 65): THIS PRODUCT MAY CONTAIN THE FOLLOWING CHEMICALS KNOWN TO THE STATE OF CALIFORNIA TO CAUSE CANCER, BIRTH DEFECTS OR OTHER REPRODUCTIVE HARM: HEXACHLOROBENZENE (<0.003%) STATE RIGHT TO KNOW: NEW JERSEY: 75-83-2, 110-54-3, 79-29-8, 75-37-6 PENNSYLVANIA: 107-83-5, 75-83-2, 110-54-3, 79-29-8, 75-37-6 MASSACHUSETTS: 107-83-5, 75-83-2, 110-54-3, 79-29-8, 75-37-6 RHODE ISLAND: 110-54-3, 75-37-6 ADDITIONAL REGULATORY INFORMATION: IN STATES WITH CONSUMER PRODUCTS VOC REGULATIONS, THIS PRODUCT IS COMPLIANT AS A SILICONE-BASED MULTI-PURPOSE LUBRICANT. SECTION 16: OTHER INFORMATION. NFPA: HEALTH 2 FLAMMABILITY 3 REACTIVITY 0 HMIS: HEALTH 2 FLAMMABILITY 3 REACTIVITY 0 PPE B PREPARED BY: MICHELLE RUDNICK CRC #: 517D/E REVISION DATE: 01/25/2008 CHANGES SINCE LAST REVISION: PROPELLANT CHANGE THE INFORMATION CONTAINED IN THIS DOCUMENT APPLIES TO THIS SPECIFIC MATERIAL AS SUPPLIED. IT MAY NOT BE VALID FOR THIS MATERIAL IF IT IS USED IN COMBINATION WITH ANY OTHER MATERIALS. THIS INFORMATION IS ACCURATE TO THE BEST OF CRC INDUSTRIES' KNOWLEDGE OR OBTAINED FROM SOURCES BELIEVED BY CRC TO BE ACCURATE. BEFORE USING ANY PRODUCT, READ ALL WARNINGS AND DIRECTIONS ON THE LABEL. CAS: CHEMICAL ABSTRACT SERVICE PPM: PARTS PER MILLION TCC: TAG CLOSED CUP PMCC: PENSKY-MARTENS CLOSED CUP PPE: PERSONAL PROTECTION EQUIPMENT TWA: TIME WEIGHTED AVERAGE OSHA: OCCUPATIONAL SAFETY AND HEALTH ADMINISTRATION ACGIH: AMERICAN CONFERENCE OF GOVERNMENTAL INDUSTRIAL HYGIENISTS NIOSH: NATIONAL INSTITUTE OF OCCUPATIONAL SAFETY & HEALTH AIHA: AMERICAN INDUSTRIAL HYGIENE ASSOCIATION NA: NOT APPLICABLE ND: NOT DETERMINED NE: NOT ESTABLISHED G/L: GRAMS PER LITER LBS./GAL: POUNDS PER GALLON STEL: SHORT TERM EXPOSURE LIMIT |
| --- | --- |