MSDS for Marvel Air Tool Oil

June 29, 2006

MATERIAL SAFETY DATA SHEET

1.    PRODUCT AND COMPANY IDENTICATION

PRODUCT NAME                   Marvel Air Tool Oil

PRODUCT CODE                   MM080, MM085, MM85R, MM086, MM088R,

MM089 MM

CHEMICAL FAMILY                Petroleum Distillates

CHEMICAL NAME                  Complex Mixture of Hydrocarbons

FORMULA                        Mixture

MANUFACTURER                        EMERGENCY TELEPHONE NUMBERS

Marvel Oil Company, Inc               Transportation:

625 Willowbrook Centre Parkway        CHEMTREC: 800-424-9300

Willowbrook, IL 60527                 Medical:

Phone: 630-455-3866                   ROCKY MTN POISON CTR: 800-332-3073

Fax: 630-455-3868

2.    POSITION/INFORMATION ON INGREDIENTS

COMPONENT                   CAS NUMBER           CONCENTRATION (wt %)

Naphthenic Hydrocarbons     64742-52-5            70  80

Mineral Spirits             08052-41-3            20  30

Chlorinated Hydrocarbons    00095-50-1            01

EXPOSURE LIMITS 8 hrs. TWA(ppm)

OSHA PEL              ACGIH TLV

Naphthenic Hydrocarbons    5 as oil mist          5 as oil mist

Mineral Spirits            100                    100

Chlorinated Hydrocarbons   25                     25

3.    HAZARDS IDENTIFICATION

POTENTIAL HEALTH EFFECTS

INHALATION: Can cause nasal and respiratory irritation, dizziness,

weakness, fatigue, nausea, headache, possible unconsciousness and

even asphyxiation.

INGESTION: Can cause gastrointestinal irritation, nausea, vomiting and

diarrhea. Aspiration into lungs can cause pneumonitis which can be fatal.

SKIN CONTACT: Prolonged or repeated contact can cause moderated

irritation, defatting or dermatitis.

EYE CONTACT: Can cause severe irritation, redness, tearing or blurred

vision.

4.    FIRST AID MEASURES

EYE: Flush with large amounts of water, lifting upper and lower eyelids

occasionally. Get medical attention.

SKIN: Thoroughly wash exposed area with soap and water. Remove

contaminated clothing. Launder before re-use.

INHALATION: Remove person to fresh air. If breathings difficult,

administer oxygen. If breathing has stopped, give artificial respiration.

Keep person warm and quiet. Call a physician.

INGESTION: Do not induce vomiting. Keep person quiet and warm. Get medical

attention. Aspiration of material into lungs can cause chemical

pneumonitis which can be fatal.

5.    FIRE FIGHTING MEASURES

FLASH POINT: 128 º F (53 º C) TCC

EXTINGUISHING MEDIA: Carbon dioxide, dry chemical, foam.

SPECIAL FIRE FIGHTING PROCEDURES: Wear self-contained breathing apparatus

with full facepiece operated with positive pressure-demand when fighting

large fires.

6.    ACCIDENTIAL RELEASE MEASURES

SPILL OR LEAK PROCEDURES: Ventilate area. Remove sources of ignition.

Prevent entry into sewers and waterways. Pick up free liquid for recycle

and/or disposal. Absorb small amounts on inert material for disposal.

7.    HANDLING AND STORAGE

STORAGE TEMPERATURE (MIN./MAX.): -40 º F (-40 º C)/ 120 º F (49 C)

SHELF LIFE: 3 years minimum when the original container is kept tightly

closed and properly stored.

SPECIAL SENSITIVITY: None.

HANDLING AND STORAGE PRECAUTIONS: Empty containers may be dangerous since

fumes may still exist. Observe precautions given for this product as

stated in this document.

8.    EXPOSURE CONTROLS/PERSONAL PROTECTION

EYE PROTECTION REQUIREMENTS: Splash goggles.

SKIN PROTECTION REQUIREMENTS: Wear chemically resistant gloves.

RESPIRATOR/VENTILATION REQUIREMENTS: Provide sufficient ventilation to

avoid exposure levels above the established TLV's.

EXPOSURE LIMITS: Not established for product as whole.

Mineral Spirits     NIOSH       100 ppm TWA

Oil Mist            OSHA           5 mg/m3

9.    PHYSICAL AND CHEMICAL PROPERTIES

PHYSICAL FORM:                       Thin liquid

COLOR:                       Red

ODOR:                        Petroleum

BOILING POINT:               Not determined

MELT / FREEZE POINT          - 60 º F ( - 51 º C)

PH:                          Not applicable

SOLUBILITY IN WATER:         Insoluble

SPECIFIC GRAVITY:            0.876 @ 60 º F (15.6 º C)

% VOLATILE BY WEIGHT:        25 %

VAPOR PRESSURE:              Not determined

VAPOR DENSITY:               Not determined

10.   REACTIVITY

STABILITY: Stable

HAZARDOUS POLYMERIZATION: Will not occur.

INCOMPATIBILITIES: Strong oxidizing agents.

DECOMPOSITION PRODUCTS: Carbon monoxide, carbon dioxide and hydrocarbons.

11.   TOXICOLOGICAL INFORMATION

ACUTE INHALATION: Aspiration into lungs can cause pneumonitis which

can be fatal.

CHRONIC INHALATION: Not determined.

ACUTE SKIN CONTACT: Prolonged or repeated contact can cause moderate

irritation, defatting or dermatitis.

CHRONIC SKIN CONTACT: Not determined.

ACUTE EYE CONTACT: Can cause severe irritation, redness, tearing or

blurred vision.

12.   ECOLOGICAL INFORMATION

No data available.

13.   DISPOSAL CONSIDERATIONS

Ignitable hazardous waste, EPA Hazardous Waste Number D001

WASTE DISPOSAL METHOD: Dispose of product in accordance with all local,

state and federal laws and regulations.

14.   TRANSPORT INFORMATION

DOT INFORMATION:

PROPER SHIPPING NAME:         Non Bulk      Not regulated

Bulk          Petroleum distillates, n.o.s.

TECHNICAL SHIPPING NAME:                    Fuel and oil additive

HAZARD CLASS:                 Non Bulk      ORM-D

Bulk          Class 3

UN NUMBER:                                  UN 1268

PRODUCT RQ (lbs):                           None

LABEL:                        Non Bulk      ORM-D

Bulk          Flammable Liquid

PLACARD:                      Non Bulk      None

Bulk          Flammable Liquid

FREIGHT CLASS BULK:                         PG III

FREIGHT CLASS PACKAGE:                      None

PRODUCT LABEL                               None

15.   REGULATORY INFORMATION

TSCA STATUS:                                All ingredients listed.

CERCLA REPORTABLE QUANTITY:                 None

SARA TITLE III:

SECTION 302 EXTREMELY

HAZARDOUS SUBSTANCES                   None

SECTION 311/312

HAZARD CATEGORIES

Acute Health                   Yes

Chronic Health                 Yes

Fire                           Yes

Reactive                       No

Sudden Release of Pressure     No

SECTION 313

CHEMICAL NAME                 CAS NUMBER             CONCENTRATION

Ortho-dichlorobenzene         00095-50-1              0 -0.25 %

RCRA STATUS: If discarded in its purchased form, this product would be an

ignitable waste with an EPA Hazardous Waste Number of D001. However,

under RCRA, it is the responsibility of the product user to determine at

the time of disposal, whether a material containing the product should be

classified as a hazardous waste. (40CFR261.20-24)

CANADIAN STATUS: All materials contained in this product are listed on the

Canadian Domestic Substances List.

EUROPEAN UNION: All materials contained in this product are listed on

EINECS.

STATE REGULATORY INFORMATION

The following chemicals are specifically listed by individual states;

other product specific health and safety data in other sections of the

MSDS may also be applicable for state requirements. For details on your

regulatory requirements you should contact the appropriate agency in your

state.

COMPONENT/

CAS NUMBER                CONCENTRATION               STATE CODE

p-dichlorobenzene         less than 150 ppm           CA

00106-46-7

CA = Material known to the state of California to cause cancer and/or

birth defects. (California Proposition 65).

16.   OTHER INFORMATION

HMIS CLASSIFICATION      Health          2

Flammability    2

Reactivity      0

PPI             B

NFPA RATING              Health          2

Fire            2

Reactivity      0

Special         None

REASON FOR ISSUE         Revise address, phone & fax numbers

PREPARED BY              Richard P. Kelly

TITLE                    Technical Manager

APPROVAL DATE            June 29, 2006

SUPERCEDES DATE          June 9, 2005

REVISION NUMBER          #07

This information is to the best of Marvel Oil Company's knowledge and

belief, accurate and reliable. However, no representation, warranty, or

guarantee is made to its accuracy, reliability or completeness. It

is the users responsibility to satisfy himself as to the suitableness and

completeness of such information for his own particular use.

Msdsmm080-3.doc

The information contained in this MSDS was obtained from current and reliable sources, however, the data is provided without any warrenty, expressed or implied, regarding its correctness or accuracy. Since the conditions or handling, storage and disposal of this product are beyond the control of Imperial Supplies LLC, Imperial will not be responsible for loss, injury, or expense arising out of the products improper use. No warranty, expressed or inferred, regarding the product described in this MSDS shall be created or inferred by any statement in this MSDS. Various government agencies may have specific regulations regarding the transportation, handling, storage, use, or disposal of this product which may not be covered by this MSDS. The user is responsible for full compliance.

For more product information by email, click here

Imperial Supplies LLC does not provide services and products outside the United States