ATL Air Tool Lubricant

=======================================================================

MATERIAL SAFETY DATA SHEET

Effective Date:  none                             Revision Date:  none

ATL Air Tool Lubricant

Code: CPN                                                            Page: 1

=======================================================================

Section 1 - Product and Company Identification

PRODUCT NAME: ATL Air Tool Lubricant

MANUFACTURER'S NAME:                     EMERGENCY TELEPHONE NUMBER

Coilhose Pneumatics                                   (908)752-5000

200 Clay Avenue

Middlesex, NJ 08846                             MISCELLANEOUS INFORMATION

=======================================================================Section 2 - Hazardous Ingredients

COMPONENTS:     W/W   HAZARD DATA(TLV, LD50, LC50, ETC.) :

Petroleum-based lubricating oil        TLV 5 mg. /meter cubed

CAS #'s:   64742-58-1 and            (as an oil mist)

64742-62-7

=======================================================================Section 3 - Hazards Identification

HAZARDOUS MATERIALS IDENTIFICATION SYSTEM  (HMIS) :

Health   Flammability    Reactivity

1           1              0

ORAL  (Acute):             LD 50  >  5 g/kg  (total body weight)

DERMAL  (Acute):           LD 50  >  3.16 g/kg  (total body weight)

EYE:                       N/E

INHALATION  (Acute):       N/E

CHRONIC, SUBCHRONIC, ETC.: N/E

Medical Conditions Aggravated by Exposure:  Unknown

This product does NOT contain any ingredients identified as carcinogenic by IRAC, NTP, or OSHA.

SARA Section 313 Status:  This material is not known to contain any

chemicals on the SARA Section 313 list at a concentration greater than

1.0 percent or carcinogenic chemical on that list at a concentration

greater then 0.1 percent.

=======================================================================Section 4 - First Aid Measures

EYE CONTACT:  If splashed into the eyes, flush with clear water for 15

minutes or until irritation subsides.  If irritation persists, call a

physician.

SKIN CONTACT:  In case of skin contact, remove contaminated clothing and wash skin thoroughly with soap and water.

INHALATION:  Vapor pressure is very low.  Vapor inhalation under ambient conditions is normally not a problem.  If overcome by vapor form hot product, immediately remove from exposure and call a physician.  If breathing is irregular or has stopped, start resuscitation; administer oxygen if available.  If overexposure to oil mist, remove from further exposure until excessive oil mist condition subsides.

INGESTION:  If ingested, do not induce vomiting.  Call a physician

immediately.

=======================================================================Section 5 - Firefighting Measures

FLASH POINT  (MINIMUM):     AUTOIGNITOIN TEMPERATURE:

365`F   Test method:  COC       N/E

NATIONAL FIRE PROTECTION ASSOCIATION (NFPA) - HAZARD IDENTIFICATION

Health     Flammability     Reactivity

1             1               0

FLAMMABLE OR EXPLOSIVE LIMITS (approximate percent by volume in air):

Estimated values:  lower  1%     upper  6%

EXTINGUISHED MEDIA AND FIRE FIGHTING PROCEDURES:  Foam, water spray (fog), dry chemical, carbon dioxide and vaporizing liquid type extinguishing agents may all be suitable for extinguishing fires involving this type product, depending on size or potential size of fire and circumstances related to the situation.  Plan fire protection and response strategy through consultation with local fire protection authorities or appropriate specialists.

The following procedures for this type of product are based on the recommendations in the National Fire Protection Association's  " Fire Protection Guide on Hazardous Materials", Eighth Edition  (1984):

Use water spray, dry chemical, foam, or carbon dioxide.  Water or foam

may cause frothing.  Use water to keep fire-exposed containers cool.

Water froth may be used to flush spills away from exposure.  Minimize

breathing gases, vapor, fumes, or decomposition products.  Use

supplied-air breathing equipment for enclosed or confined spaces or as

otherwise needed.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  N/A

"EMPTY" CONTAINER WARNING:  Empty containers retain residue (liquid or

vapor) and can be dangerous.  DO NOT PRESSURIZE, WELD, CUT BRAZE,

SOLDER, DRILL, GRIND OR EXPOSE SUCH CONTAINERS TO HEAT, FLAME, SPARKS,

OR OTHER SOURCES OF IGNITION; THEY MAY EXPLODE AND CAUSE INJURY

OR DEATH.  Do not attempt to clean since residue is difficult to

remove.  "Empty" drums should be completely drained, properly bunged,

and returned to a drum reconditioner.  All other containers should be

disposed of in an environmentally safe manner and in accordance with

government regulations.  For work on tanks refer to Occupational Safety

and Health Administration regulations, ANSI Z49.1, and other vernmental

and industrial references pertaining to cleaning, repairing, welding,

or other contemplated operations.

EXPOSURE LIMIT FOR TOTAL PRODUCT:  5 mg/cubic meter for oil mist in air

BASIS:  OSHA Regulation 29 CFR 1910. 1000

VARIABILITY AMONG INDIVIDUALS:  Health studies have shown that many

petroleum hydrocarbons and synthetic lubricants pose potential human

health risks which vary from person to person.  As a precaution,

exposure to liquids, vapors, mists, or fumes should be minimized.

EFFECTS OF OVEREXPOSURE  (Signs and symptoms of exposure):

Prolonged or repeated skin contact with this product tends to remove

=======================================================================Section 5 - Firefighting Measures - Continued

skin oils possibly leading to irritation and dermatitis; however, based

on human experience and available toxicological data, this product is

judged to be neither a "corrosive" nor an "irritant" by OSHA criteria.

Product contacting the eye may cause irritation.

Product has low order of oral and dermal toxicity.  Possible aspiration

hazard.  Induced vomiting may cause aspiration of product into the

lungs.  (See Emergency First Aid Section)

=======================================================================Section 6 - Environmental Release Measures

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED:

Keep product out of sewers and watercourses by diking or impounding.

Absorb with sand or inert material.  Sweep or scoop up and remove.

Prevent spread of spill.  Advise authorities if product has entered or

may enter sewers, watercourses or extensive land areas.  Assure

conformity with local regulations.

=======================================================================Section 7 - Handling and Storage

Not available

=======================================================================

Section 8 - Exposure Controls/Personal Protection

VENTILATION:  (Always maintain below permissible exposure limits.)

Use local exhaust to capture vapor, mist, or fumes, if necessary.

Provide greater than 60 feet per minute hood face velocity for confined

spaces.  Provide ventilation sufficient to prevent exceeding

recommended exposure limit or buildup of explosive concentrations of

vapor in air.

RESPIRATORY PROTECTION:  (Use only NIOSH approved equipment.)

Normally not needed at ambient temperatures.  Use supplied air

respiratory protection in confined or enclosed spaces, if needed.  Use

filter, dust, fume, or mist respirator type under misting conditions.

Use can or cartridge; gas or vapor respirator type under conditions

exceeding TWA standard.

PROTECTIVE GLOVES:  Use chemical-resistant gloves, if needed, to avoid

prolonged or repeated skin contact.

EYE PROTECTION:  Use splash goggles or face shield when eye contact may

occur.

OTHER PROTECTIVE EQUIPMENT:  Use chemical-resistant apron or other

impervious clothing, if needed, to avoid contaminating regular clothing

which could result in prolonged or repeated skin contact.

WORK PRACTICES/ENGINEERING CONTROLS:  Keep containers closed when not in use.  Do not handle near heat, sparks, flame, or strong oxidants.

PERSONAL HYGIENE:  Minimize breathing vapor, mist, or fumes.  Avoid

prolonged or repeated contact with skin.  Remove contaminated clothing;

launder or dry-clean before reuse.  Remove contaminated shoes and

thoroughly clean before reuse; discard if oil-soaked.  Cleanse skin

thoroughly after contact, before breaks and meals, and at end of work

period.  Product is readily removed from skin by waterless hand

cleaners followed by washing thoroughly with soap and water.

=======================================================================

Section 9 - Physical and Chemical Properties

PRODUCT APPEARANCE AND ODOR: Amber liquid, petroleum odor

CHEMICAL FAMILY:  Petroleum hydrocarbon

SYNONYMS:  Petroleum-based lube oil

EMERGENCY TELEPHONE:  (908) 752-5000

The following data are approximate or typical values and should not be used for precise design purposes.

BOILING RANGE             :  Wide range

VAPOR PRESSURE            :  < 0.1 @ 38`C/100`F

SPECIFIC GRAVITY (25C/25C):  (WATER = 1)< 1.0

VAPOR DENSITY (AIR = 1)   :  > 8

MOLECULAR WEIGHT          :  Wide range

PERCENT VOLATILE BY VOLUME:  Negligible

EVAPORATION RATE @ 1 ATM. AND 25C (77F)(n-BUTYL ACETATE = 1):  < 1.0

SOLUBILITY IN WATER @ 1 ATM. and 25C (77F):  Negligible

POUR, CONGEALING OR MELTING POINT:  N/E

FREEZING POINT            :  N/E

=======================================================================Section 10 - Stability and Reactivity

This product is stable and will NOT react violently with water.  Hazardous polymerization will not occur.  Avoid contact with strong oxidants such as liquid chlorine, concentrated oxygen, sodium hypochlorite or calcium hypochlorite.

DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS:

Fumes, smoke, carbon monoxide, oxides of sulfur, and other decomposition products, in case of incomplete combustion.

CONDITIONS TO AVOID:  Open flames.

=======================================================================Section 11 - Toxicological Information

Not available

=======================================================================Section 12 - Ecological Information

Not available

=======================================================================Section 13 - Disposal Considerations

WASTE DISPOSAL METHOD:  (Consult federal, state, or local authorities

for proper disposal procedures.)  Assure conformity with applicable

disposal regulations.  Dispose of absorbed material at an approved

waste site or facility.

=======================================================================Section 14 - Transport Information

TRANSPORTATION INCIDENT INFORMATION:

ICC:  Compound or lubricant.  Metal cutting, drawing or drilling.  Dry,

liquid, or paste.  NOI

=======================================================================Section 15 - Regulatory Information

Not available

=======================================================================Section 16 - Other Information

The above information is accurate to the best of our knowledge.  However,

since data, safety standards, and government regulations are subject to

change and the conditions of handling and use, or misuse are beyond our

control, seller makes no warranty, either express or implied, with respect

to the completeness or continuing accuracy of the information contained

herein and disclaims all liability for reliance thereon.  User should

satisfy himself that he has all current data relevant to his particular use.