MSDS Sheet for Fluorescent Gas Leak Detector

NU-CALGON WHOLESALER, INC.

MATERIAL SAFETY DATA SHEET

| SECTION 1 - CHEMICAL PRODUCT AND COMPANY IDENTIFICATION |  |
| --- | --- |

COMPANY NAME: NU-CALGON WHOLESALER, INC.

STREET ADDRESS: 2008 ALTOM COURT
CITY: ST. LOUIS
STATE: MO
POSTAL CODE: 63146-4151

PHONE NUMBER:
(314) 469-7000
(800) 554-5499

CHEMTREC: (800) 424-9300

PRODUCT NAME: FLUORESCENT GAS LEAK DETECTOR

PRODUCT NUMBER: 4184

PRODUCT USE: GAS LEAK DETECTOR

EPA REGISTRATION #: N/AV

LAST UPDATE: 1/10/08

| SECTION 2 - COMPOSITION/INFORMATION ON INGREDIENTS |  |
| --- | --- |

HAZARDOUS INGREDIENTS     % BY WT.   CAS NUMBER     TLV              PEL

ETHYLENE GLYCOL           15-25%      107-21-1      50 PPM           50 PPM
                                                    (ACGIH CEILING)

SURFACTANT MIXTURE         1-5%      PROPRIETARY    N/AV             N/AV

WATER                     60-80%     7732-18-5      N/AV             N/AV

OTHER INGREDIENTS ARE PRESENT AT LESS THAN 1% OR ARE TRADE SECRETS.

| SECTION 3 - HAZARD IDENTIFICATION |  |
| --- | --- |

EMERGENCY OVERVIEW:

CONTAINS ETHYLENE GLYCOL:
DO NOT USE IN FOOD, DRUG, COSMETIC OR POTABLE WATER APPLICATIONS.

HARMFUL OR FATAL IF SWALLOWED - ASPIRATION HAZARD IF SWALLOWED - CAN ENTER
LUNGS AND CAUSE DAMAGE. CAN CAUSE KIDNEY AND LIVER DAMAGE IF SWALLOWED. DO
NOT GET IN EYES, ON SKIN OR CLOTHING. AVOID BREATHING SPRAY OR MIST. USE
ONLY WITH ADEQUATE PERSONAL PROTECTION EQUIPMENT.

POTENTIAL HEALTH EFFECTS:

EYES:
PROLONGED OR REPEATED CONTACT WITH EYES MAY CAUSE IRRITATION, REDDENING AND
DRYING.

SKIN:
PROLONGED OR REPEATED CONTACT WITH SKIN MAY CAUSE IRRITATION, REDDENING,
DRYING AND CRACKING OF SKIN. ABSORPTION THROUGH SKIN POSSIBLE. SEE WARNINGS
FOR CHRONIC OR LONG TERM SKIN EXPOSURE.

INGESTION:
MATERIAL IS TOXIC. HARMFUL OR FATAL IF SWALLOWED - MAY CAUSE NAUSEA,
CENTRAL NERVOUS SYSTEM DAMAGE, HEADACHE, WEAKNESS, CONFUSION, SLURRED
SPEECH, LOSS OF COORDINATION, CARDIOPULMONARY EFFECTS, CARDIAC FAILURE,
COMA AND CONVULSIONS. ASPIRATION HAZARD IF SWALLOWED - CAN ENTER LUNGS AND
CAUSE DAMAGE. CAN CAUSE KIDNEY AND LIVER DAMAGE IF SWALLOWED. LETHAL DOSE
IS APPROXIMATELY 20 OUNCES BASED ON TOXICITY OF ETHYLENE GLYCOL.

INHALATION:
VAPORS OR MISTS IN EXCESS OF PERMISSABLE CONCENTRATIONS, OR IN UNUSUALLY
HIGH CONCENTRATIONS AS FROM EXPOSURE IN POORLY VENTILATED AREAS OR CONFINED
SPACES MAY CAUSE IRRITATION OF NOSE AND THROAT, HEADACHE, NAUSEA, AND
DROWSINESS. PROLONGED OR REPEATED OVEREXPOSURE MAY CAUSE INGESTION EFFECTS.

CHRONIC EXPOSURE:
REPEATED INGESTION OR OVEREXPOSURE TO ETHYLENE GLYCOL MAY CAUSE BIRTH
DEFECTS BASED ON ANIMAL DATA AS WELL AS KIDNEY AND LIVER DAMAGE, AND
NERVOUS SYSTEM DAMAGE. REPEATED CONTACT WITH SKIN MAY RESULT IN ABSORPTION
OF HARMFUL AMOUNTS.

CARCINOGENICITY: N/AV

MEDICAL CONDITIONS AGGRAVATED BE EXPOSURE:
PREEXISTING KIDNEY DISORDERS, LUNG DISORDERS, AND DERMATITIS.

| SECTION 4 - FIRST AID MEASURES |  |
| --- | --- |

EYES:
IMMEDIATELY FLUSH WITH WATER FOR AT LEAST 15 MINUTES AND CALL A PHYSICIAN.
HOLD EYELIDS APART WHILE FLUSHING TO RINSE ENTIRE SURFACE.

SKIN:
REMOVE CONTAMINATED CLOTHING. WASH WITH LARGE AMOUNTS OF SOAP AND WATER
FOR SEVERAL MINUTES. CONSULT A PHYSICIAN IN THE CASE OF ANY PROLONGED
IRRITATION

INGESTION:
IF PERSON IS CONSCIOUS AND CAN SWALLOW, IMMEDIATELY GIVE 2 GLASSES OF WATER
AND INDUCE VOMITING. AFTER VOMITING OCCURS, GIVE FLUIDS AGAIN.
CONTACT A PHYSICAN IMMEDIATELY!

INHALATION:
REMOVE TO FRESH AIR. START ARTIFICIAL RESPIRATION IF NECESSARY. OXYGEN MAY
BE ADMINISTERED. CALL A PHYSICIAN.

| SECTION 5 - FIREFIGHTING MEASURES |  |
| --- | --- |

FLASH POINT: >220 DEG. F

AUTOIGNITION TEMP:
N/AV DEG. C
N/AV DEG. F

HAZARDOUS PRODUCTS OF COMBUSTION:
BURNING CAN PRODUCE OXIDES OF CARBON AND OTHER SUBSTANCES.

FLAMMABLE LIMITS IN AIR: N/AV

EXTINGUISHING MEDIA:
THIS PRODUCT IS NOT COMBUSTIBLE. WATER SPRAY, FOAM, CO2 OR DRY CHEMICALS
MAY BE USED IN AREAS WHERE THIS PRODUCT IS STORED.

FIRE AND EXPLOSION HAZARDS:
DO NOT ENTER CONFINED FIRE-SPACES WITHOUT PROTECTIVE CLOTHING AND
SELF-CONTAINED AIR SUPPLY.

SPECIAL FIREFIGHTING PROCEDURES:
DO NOT ENTER CONFINED FIRE-SPACES WITHOUT PROTECTIVE CLOTHING AND
SELF-CONTAINED AIR SUPPLY.

| SECTION 6 - ACCIDENTAL RELEASE MEASURES |  |
| --- | --- |

SPILL OR LEAK:
SAFELY STOP SPILL AT SOURCE. CONTAIN SPILL BY DIKING WITH SOIL OR OTHER
INERT MATERIAL. MOP, PUMP OR ABSORB WITH INERT MATERIAL AND RECLAIM INTO
SOUND CONTAINER FOR PROPER DISPOSAL. PREVENT ENTRY INTO SEWERS AND
WATERWAYS.

| SECTION 7 - HANDLING AND STORAGE |  |
| --- | --- |

HANDLING PROCEDURES AND EQUIPMENT:
MINIMUM FEASIBLE HANDLING TEMPERATURES SHOULD BE MAINTAINED. WASH
THOROUGHLY AFTER HANDLING. AVOID CONTACT WITH EYES, SKIN AND CLOTHING.
AVOID BREATHING MISTS OR VAPORS. HARMFUL OR FATAL IF SWALLOWED.

STORAGE REQUIREMENTS:
KEEP CONTAINER CLOSED WHEN NOT IN USE. FOR INDUSTRIAL USE ONLY.

| SECTION 8 - EXPOSURE CONTROLS/PERSONAL PROTECTION |  |
| --- | --- |

RESPIRATORY PROTECTION:
USE WITH ADEQUATE VENTILATION. SPECIFIC USE CONDITIONS (SPRAYING/CONFINED
SPACES) WHERE REGULATORY LIMITS FOR ETHYLENE GLYCOL ARE EXCEEDED, MAY
REQUIRE LOCAL EXHAUST VENTILATION TO PREVENT RELEASE OF MIST &/OR VAPORS
INTO WORK ENVIRONMENT. IF VENTILATION IN NOT ADEQUATE, USE NIOSH/MSHA
APPROVED RESPIRATOR.

EYE PROTECTION: SAFETY GLASSES/GOGGLES

PROTECTIVE CLOTHING:
CHEMICAL RESISTANT GLOVES RECOMMENDED. NO SPECIAL REQUIREMENTS - WORK
CLOTHING APPROPRIATE TO MINIMIZE CONTACT/REDUCE EXPOSED SKIN AREA.

EXPOSURE GUIDELINES:

ETHYLENE GLYCOL:
TLV: 50 PPM. EYE WASH STATION AND SAFETY SHOWER IN HANDLING AREA.

SPECIFIC ENGINEERING CONTROLS (SUCH AS VENTILATION, ENCLOSED PROCESS):
INSURE ADEQUATE VENTILATION TO CONTROL ETHYLENE GLYCOL AIRBORNE
CONCENTRATION BELOW TLV OF 50 PPM. EYE WASH STATION AND SAFETY SHOWER IN
HANDLING AREA.

| SECTION 9 - PHYSICAL AND CHEMICAL PROPERTIES |  |
| --- | --- |

PHYSICAL FORM: VISCOUS LIQUID

FREEZING POINT:
-9 DEG. C
15 DEG. F

% VOLATILE BY WEIGHT: APPROX. 78%

COLOR: FLUORESCENT GREEN-YELLOW

VAPOR DENSITY (AIR=1): NOT DETERMINED

EVAPORATION RATE VS. H2O: ABOUT THE SAME

ODOR: CHARACTERISTIC MILD ODOR

VAPOR PRESSURE: N/AV

SPECIFIC GRAVITY (H2O=1): 1.0 (+/-0.05)

BOILING POINT: 100 DEG. C/212 (IBP) DEG. F

SOLUBILITY IN WATER: COMPLETE

pH (CONCENTRATE): 8.3 (+/-0.5)

| SECTION 10 - STABILITY AND REACTIVITY |  |
| --- | --- |

CHEMICAL STABILITY: STABLE

HAZARDOUS POLYMERIZATION: NONE

INCOMPATIBILITIES:
STRONG ACIDS/OXIDIZERS. DO NOT MIX WITH CHLORINATED DETERGENTS (BLEACH).

REACTIVE CONDITIONS TO AVOID: N/AV

DECOMPOSITION PRODUCTS:
BURNING CAN PRODUCE OXIDES OF CARBON, AS WELL AS ALDEHYDES, KEYTONES, AND
OTHER SUBSTANCES.

| SECTION 11 - TOXICOLOGICAL INFORMATION |  |
| --- | --- |

HAZARDOUS       CAS #     EINECS #   LD50 OF INGREDIENT  LC50 OF INGREDIENT
INGREDIENTS                          (SPECIFY SPECIES)   (SPECIFY SPECIES)

ETHYLENE        107-21-1  203-473-3  1000 MG/KG (HUMAN)  130 MG/M3
GLYCOL                                                   (MOUSE, 2HR)

| SECTION 12 - ECOLOGICAL INFORMATION |  |
| --- | --- |

HAZARDOUS INGREDIENTS           AQUATIC TOXICITY DATA

ETHYLENE GLYCOL                 LC50 (96HR) RAINBOW TROUT: 41,000 MG/L

| SECTION 13 - DISPOSAL CONSIDERATIONS |  |
| --- | --- |

WASTE DISPOSAL:
DISPOSE OF IN AN APPROVED WASTE FACILITY ACCORDING TO FEDERAL, STATE AND
LOCAL REGULATIONS.

| SECTION 14 - TRANSPORTATION INFORMATION |  |
| --- | --- |

SPECIAL SHIPPING INFORMATION: N/AV

PURVIEW   PROPER SHIPPING NAME       UN NUMBER  PACKING GROUP  HAZARD CLASS

DOT       NOT REGULATED IN
(LAND)    CONTAINERS 55 GAL OR LESS

IMO       N/AV
(WATER)

ICAO      N/AV
(AIR)

| SECTION 15 - REGULATIORY INFORMATION |  |
| --- | --- |

WHMIS CLASSIFICATION (WORKPLACE HAZARDOUS MATERIAL INFORMATION SYSTEM):
CLASS D - DIVISION 2A - TERATOGEN

SARA TITLE III (SUPERFUND AMENDMENTS & REAUTHORIZATION ACT):
CONTAINS ETHYLENE GLYCOL AT 15-25% BY WEIGHT.

OSHA (OCCUPATIONAL SAFETY & HEALTH ADMINISTRATION):
OSHA HAZARDOUS - ACUTE & CHRONIC HAZARD

TSCA (TOXIC SUBSTANCE CONTROL ACT): ALL INGREDIENTS ARE TSCA REGISTERED.

VOC (VOLATILE ORGANIC COMPOUNDS): LESS THAN 1%

CPR (CANADIAN CONTROLLED PRODUCTS REGULATIONS):
THIS PRODUCT HAS BEEN CLASSIFIED IN ACCORDANCE WITH THE HAZARD CRITERIA OF
THE CONTROLLED PRODUCTS REGULATIONS

EINECS (EUROPEAN INVENTORY OF EXISTING COMMERCIAL CHEMICAL SUBSTANCES):
N/AV

DSL/NDSL (CANADIAN DOMESTIC SUBSTANCE LIST) (NON-DOMESTIC SUBSTANCE LIST):
NOT ALL INGREDIENTS WITHIN THIS PRODUCT ARE ON THE DSL AND/OR NDSL.

CERCLA (COMPREHENSIVE RESPONSE COMPENSATION & LIABILITY ACT): >5,000 LBS

IDL (CANADIAN INGREDIENT DISCLOSURE LIST):
ETHYLENE GLYCOL IS LISTED. OTHER INGREDIENTS ARE BELOW DISCLOSURE.

NFPA (HMIS) RATING (HAZARDOUS MATERIALS IDENTIFICATION SYSTEM):
HEALTH              2*
FLAMMABILITY        0
REACTIVITY          0
PERSONAL PROTECTION X

| SECTION 16 - OTHER INFORMATION |  |
| --- | --- |

THE ANTIDOTES FOR ETHYLENE GLYCOL POISONING ARE ETHANOL OR FOMEPIZOLE;
ANTIDOTAL TREATMENT FORMS THE MAINSTAY OF MANAGEMENT FOLLOWING INGESTION.
ETHANOL (USUALLY GIVEN IV AS A 5 OR 10% SOLUTION IN 5% DEXTROSE AND WATER,
BUT, ALSO SOMETIMES GIVEN IN THE FORM OF A STRONG SPIRIT SUCH AS WHISKY,
VODKA OR GIN) ACTS BY COMPETING WITH ETHYLENE GLYCOL FOR THE ENZYME ALCOHOL
DEHYDROGENASE THUS LIMITING THE FORMATION OF TOXIC METABOLITES.

THE INFORMATION CONTAINED HEREIN IS BASED ON THE DATA AVAILABLE TO US AND
IS BELIEVED TO BE CORRECT. HOWEVER, NU-CALGON WHOLESALER INC. MAKES NO
WARRANTY, EXPRESSED, OR IMPLIED, REGARDING THE ACCURACY OF THIS DATA OR THE
RESULTS TO BE OBTAINED FROM THE USE THEREOF. NU-CALGON WHOLESALER INC.
ASSUMES NO LIABILITY FOR INJURY FROM THE USE OF THE PRODUCT DESCRIBED
HERIN.