| # TITLE: TDI-Brooks – High-Res Deployment | # DATE: 12/07/05 |
| --- | --- |
| ### STAFF: 3 Man Crew: Cameron Walker (Geophysicist), Shane Dunn (Surveyor), Vincent Myers (Surveyor) | # Dept: # R/V GeoExplorer |

# REQUIRED PPE: HH, SS, PFD by all on deck, clip-in safety harness by anyone on platform.

| ## STEPS | ## PROCEDURES | ## POTENTIAL HAZARDS | RECOMMENDED ACTION OR PROCEDURE |
| --- | --- | --- | --- |
| Site Arrival Assume deployment positions Confirm Rigging Begin Deployment (aft) Air gun firing Recording/Acquisition | Navigator selects location, display and scale, confirms location, and notifies deck when on station. Assign and review deck deployment/retrieval positions: Air gun array and hydrophone streamer both located on aft deck, center and port side. Establish communications between crew, navigation, bridge, and team member in high-res lab if applicable. Deck team members will be on station at aft of vessel. Acquisition team member will inspect all connections and tow points on both air gun sled and streamer. All air hose connections at the compressors and air banks will be inspected. Correct pressures will be confirmed in both air bank and regulator. All ropes, air hoses, and cables will be positioned so as to be free of obstructions and clear of crew members. Party Chief notifies Bridge that deployment is to start, then directs deployment. Begin with gun sled deployment at nearly idle speed. Guns are to be lifted with aft chain- fall and lowered into water just aft of stern. Both tow line (rope) and control cable/air lines will be deployed from aft deck to a distance of 12 meters aft of stern, port side. Gun tow point will be aft port side. Streamer is to be deployed from reel, center aft. Tail buoy will enter water first followed by tail end of streamer. Forward way must be maintained during streamer deployment. Once streamer is fully deployed a tow point will be established on port side aft. Once located at the proper offset, 12 meters aft of stern, commencement of shooting will begin. Initial shots will be used to synchronize firing of the array. Once source and receivers are deployed, under tow at appropriate locations, offsets, and velocity, recording will commence Proceed to Recovery. | Cable under load Possible MOB Possible swinging weight | While tension is on gun array lifting cable be sure to remain from beneath weight, mind foot placement Make sure all deck personnel are well clear of deploying hoses and or cables. Firm hand control must be maintained on cable, other non- essential personnel stand clear. . |