| # TITLE: TDI-Brooks – High-Res Recovery | # DATE: 12/07/05 |
| --- | --- |
| ### STAFF: Cameron Walker (Geophysicist), Shane Dunn (Surveyor), and Vincent Myers (Surveyor) | # Dept: # R/V GeoExplorer |

# REQUIRED PPE: HH, SS, PFD by all on deck, clip-in safety harness by anyone on platform.

| ## STEPS | ## PROCEDURES | ## POTENTIAL HAZARDS | RECOMMENDED ACTION OR PROCEDURE |
| --- | --- | --- | --- |
| Streamer pullout Retrieval of gun sled Wash-down | Forward way must be maintained during streamer recovery to prevent sinking. Streamer deck cable will be disconnected from streamer head. Starting at head connection streamer will be recovered directly onto reel. One man manually recovering the streamer and another turning the wheel, the wheel is not to bear drag load. Tail buoy will be hoisted on deck by hand. Following the complete recovery of streamer, gun recovery will commence. Initial gun recovery will consist of manual pulling in gun sled (by hand). To achieve this the vessel must not be under way. Once gun sled is sufficiently close to aft deck, a connection will be made between the sled and the aft chain fall. The final recovery will be made via the chain fall. The gun sled will be set on the aft deck where it will be secured. . | Possible MOB Possible rope burns Possible MOB Working under loaded wire Slippery surface | .Be aware of proximity to vessel’s edge . Wear gloves when handling any cable or rope Be aware of proximity to vessel’s edge . Be sure all deck personnel are in safe positions. Make sure there is a good footing wherever you need to be to wash the rig. Maintain clean deck absent of mud. |