| # TITLE: TDI-Brooks – SideScan/Mag Deployment | # DATE: 12/07/05 |
| --- | --- |
| ### STAFF: Survey crew and deck crew | # Dept: |

# REQUIRED PPE: HH, SS, PFD by all on deck, clip-in safety harness by anyone on platform.

| ## STEPS | ## PROCEDURES | ## POTENTIAL HAZARDS | RECOMMENDED ACTION OR PROCEDURE |
| --- | --- | --- | --- |
| Site Arrival Assume deployment / positions Confirm Rigging Begin Deployment or retrieval (aft) Begin Deployment (vertical) Connect the cable vice | Navigator selects location, display and scale, confirms location, and notifies deck when on station and gives water depth to Deck Chief. Assign and review deck deployment/retrieval positions: Winch, A-frame, Establish communications between crew, navigation, and Bridge. Notify fellow crew member whenever going out on deck always use the “Buddy System” Identify any potential hazards advise your immediate supervisor Get Deck Chief approval of deployment/recovery equipment rigging. Confirm proper rigging of the cables and fish and that all equipment is working properly, and the launch path is clear Inform the Captain that you are launching the fish and have him pick up speed to 3.5 Knots Lift the Fish and pick up slack in the cable as you approach the back deck. Place the fish over the stern net and lower in the water. Pay out cable until the fish can track bottom Once the fish is at the proper depth attach the cable vice to the fish | Surface obstructions Wet or oily surfaces Occasional spills Weather hazards Loose rugs and mats Walking surfaces that do not have the same degree of traction in all areas Obstructed views Clutter Wrinkled carpet, mats Uncovered cables Drawers open Uneven walking surfaces (including stairs) Falling to step firmly on elevated surface Poor lighting Poor housekeeping Slippery surfaces Cable under intermittent loads Pinch points Possible MOB Possible swinging trigger arm. Possible cable burrs Possible swinging weight | Travel around the survey area and get locations of possible obstructions. Make sure you have enough room to lift safely. Clear a path from the survey fish to the launch potion before lifting it. Use slow and smooth movements. Keep you body facing the survey fish while you lift it. Lift with your legs, bend your knees to pick up the load, not you back. Keep your back straight. Look around before you lift and look around as you carry. Make sure you can see where you are walking. Not using fall protection devices Make sure all deck personnel are well clear of moving equipment. No crossing of rails or cables Mind hand placement Make sure platform crew is properly clipped in to harnesses While tension is increasing on main line, be sure to stand back. Wear gloves when handling any wire. |